Valve Controller USER MANUAL

# Board image

Figure 1 Board Image

# Board layout (top view)

Figure 2 Board Layout

Figure 3 Board Description

# Valve IO:

- From left to right on Figure 3,: M1 M2 M3 M4 are motor ports.

- Each motor port has six pins: M+, M-, ENC – Encoder, SA – Sensor A, GND, SB – Sensor B.

- This board supports two types of motors:

  - M1, M2: Support full control valve (blue valve), including direction control and stop –sensor feedback. This type needs five pins to operate.

  - M3, M4: Support orange valve. Only need 2 pins: M+ M-

# Power supply:

- Support 16V-DC input and 12V Battery.

# Button:

- The Valve Controller has three button

  - BTN1: for Z-wave communication.

  - BTN2: Long press to open all valve.

  - BTN3: Long press to close all valve.

# Z-wave communication:

- This board supports Z-wave as a node.

- To enter Include/Exclude mode, long press SW1 until LED D9 blink.

- To reset Z-wave, long press SW1 until LED D9 blink and waiting for D9 blink slower.

- After connected, the valves can be control over Z-wave PC Controller program.

# How to use Z- wave PC Controller

- After Z-wave Controller (Z-Stick) is connected to computer, open Z-Wave PC Controller.

Figure 4 Zwave PC Controller UI

- Go to Network Management, click Add. Controller will wait for the node information, long press SW1 until LED D9 on the Valve Controller Board blink  The valve starts to be included into the network.

Figure 5 Add node to PC Controller

- If the inclusion completed, the program will show somethings as below:

Figure 6 Add node complete

- To control the valve, select valve ID -> select SWITCH_BINARY -> Select SWITCH_BINARY_SET -> Select Valve Sate (ON/OFF) -> Send command.

Figure 7 Control the valves using PC Controller

- You can see log by clicking Show Log.

Figure 8 Logger

s

# Leds:

- LED will indicate button processes.

# Programming port: Figure 6

- Connect to STLinkV2 to debugger.

Figure 9: Board running

# Wi-fi Communication:

# Schedule

# GSM Communication

# RS485 Communication